# Berreman

F# implementation of the Berreman matrix method for analyzing electromagnetic wave propagation through stratified (layered) optical media.

## Overview

The Berreman matrix is a 4x4 transfer matrix method used in computational electromagnetics to calculate reflection and transmission coefficients in multilayer thin films and anisotropic crystals. This library provides a comprehensive, type-safe implementation suitable for research and engineering applications in thin film optics, crystal optics, polarization analysis, and related fields.

## Features

- Full Berreman 4x4 matrix implementation for anisotropic and magneto-optically active media

- Support for arbitrary optical properties: electric permittivity (epsilon), magnetic permeability (mu), and magneto-electric coupling (rho)

- Multilayer thin film calculations with arbitrary number of layers

- Thick plate and wedge substrate handling with multiple internal reflections

- Dispersive materials with wavelength-dependent optical properties

- Complete polarization analysis: Stokes parameters, Jones vectors, Mueller matrices, ellipticity, azimuth

- Active (magneto-optic) crystals: planar, cubic (point group 23), type 3/4/6, and type 32/42/62 crystals

- Gaussian beam support via FFT-based propagation

- Type-safe units of measure (nm, um, mm, meter)

- Interactive visualization with Plotly.NET charts

- Comprehensive test suite with property-based testing

## Installation

### Requirements

- .NET 9.0 SDK

- x64 platform

### Building

dotnet build Berreman.sln -c Release

## Project Structure

Berreman/
├── Berreman/                    # Core library (Softellect.Berreman.Core)
│   └── Berreman/
│       ├── MathNetNumericsMath.fs   # Math abstractions and wrappers
│       ├── FourierTransform.fs      # FFT for Gaussian beams
│       ├── MatrixExp.fs             # Matrix exponential (Pade approximation)
│       ├── Constants.fs             # Physical constants and units
│       ├── Geometry.fs              # Vector/matrix types, rotations
│       ├── MaterialProperties.fs    # Optical properties (eps, mu, rho)
│       ├── Fields.fs                # Electromagnetic field definitions
│       ├── Media.fs                 # Optical media and systems
│       ├── BerremanMatrix.fs        # Core 4x4 matrix implementation
│       ├── Solvers.fs               # Reflection/transmission solvers
│       ├── Dispersion.fs            # Wavelength-dependent properties
│       └── FieldFunctions.fs        # Field analysis functions
├── OpticalProperties/           # Material definitions (Softellect.Berreman.OpticalProperties)
│   ├── Standard.fs                  # Standard materials (glass, crystals)
│   ├── Active.fs                    # Magneto-optic active crystals
│   └── Dispersive.fs                # Dispersive materials (Langasite, Silicon)
├── Analytics/                   # Visualization and analysis (Softellect.Berreman.Analytics)
│   ├── Variables.fs                 # Ranged variables for parametric studies
│   ├── StandardLightVariables.fs    # Common light configurations
│   ├── StandardSystems.fs           # Pre-defined optical systems
│   ├── Charting.fs                  # Plotly.NET integration
│   └── Examples/                    # Example scripts (see below)
├── BerremanTests/               # Test suite (xUnit, FsCheck)
├── BerremanRunner/              # Console application
└── BerremanModelGenerator/      # Model generation utility

## Core Concepts

### Optical Properties

Materials are defined by three 3x3 complex tensors: - Eps (epsilon): Electric permittivity tensor - Mu: Magnetic permeability tensor - Rho: Magneto-electric coupling tensor (for active crystals)

// Isotropic glass with refractive index 1.52
let glass = OpticalProperties.fromRefractionIndex (RefractionIndex 1.52)

// Uniaxial crystal
let uniaxial = OpticalProperties.uniaxialCrystal e11 e33

// Active planar crystal with magneto-electric effect
let active = OpticalProperties.planarCrystal e11 e33 g12

### Optical Systems

An optical system consists of: - Upper medium: Semi-infinite medium where light originates - Films: List of thin film layers with thickness - Substrate: Optional thick plate or wedge - Lower medium: Semi-infinite exit medium

let system = {
    description = Some "Multilayer coating"
    upper = OpticalProperties.vacuum
    films = [
        { properties = glass; thickness = Thickness.nm 100.0 }
        { properties = vacuum; thickness = Thickness.nm 150.0 }
    ]
    lower = OpticalProperties.vacuum
}

### Incident Light

Light is characterized by: - Wavelength: With unit safety (nm, um, mm) - Incidence angle: From normal (0 degrees) to grazing - Polarization: s, p, or arbitrary angle - Ellipticity: For elliptically polarized light

let light = {
    wavelength = WaveLength.nm 600.0
    incidenceAngle = IncidenceAngle.normal
    polarization = Polarization.s
    ellipticity = Ellipticity.zero
}

### Output Functions

Available output quantities: - R, T: Total reflectance and transmittance - Rs, Rp, Ts, Tp: s- and p-polarized components - Is, Ip: Incident intensity components - AzimuthR, AzimuthT: Polarization azimuth angles - EllipticityR, EllipticityT: Polarization ellipticity

## Examples

The Analytics/Examples/ folder contains F# script files demonstrating various optical scenarios. To run an example:

- Build the solution in Release mode

- Open the .fsx file in an F# interactive environment (FSI)

- Execute the script

### Glass.fsx

Simple glass substrate and film analysis

Demonstrates the difference between treating glass as a thin film vs. a thick substrate. Compares reflectance and transmittance as functions of incidence angle for a glass layer (n=1.78) at 600nm wavelength.

Key features: - Thin film interference effects vs. incoherent thick plate behavior - Incidence angle dependence (0 to 90 degrees) - Wavelength dependence (300-700nm)

### Glass_Dispersive.fsx

Dispersive glass with wavelength-dependent refractive index

Uses a Sellmeier-type dispersion formula for the ordinary refractive index:

n^2 = 1 + 1.43*lambda^2/(lambda^2 - 0.073^2) + 0.65*lambda^2/(lambda^2 - 0.12^2)

Demonstrates how to define custom dispersive materials and plot refractive index vs. wavelength.

### MultilayerThinFilm.fsx

Multilayer dielectric coating (anti-reflection)

A 40-layer alternating glass/vacuum quarter-wave stack designed for 600nm. Each layer is lambda/(4n) thick for its material: - Glass layers (n=1.52): 98.7nm - Vacuum layers (n=1.0): 150nm

Shows characteristic interference patterns in reflectance vs. wavelength and incidence angle.

### MultilayerThinFilm_02.fsx

High-reflectance multilayer mirror

100 layer pairs of alternating high/low index materials (n=1.75/1.50) on glass substrate. Demonstrates: - High reflectance at design wavelength - Spectral bandwidth engineering - Angular dependence of multilayer mirrors

### MultilayerThinFilm_EUV.fsx

EUV (Extreme Ultraviolet) multilayer mirror

100 Mo/Si layer pairs for 10.6nm (EUV) wavelength using complex refractive indices: - Molybdenum: absorption and phase shift at EUV - Silicon: spacer layers

Used in EUV lithography mirrors and X-ray optics.

### MultilayerThinFilm_EUV__002.fsx

Custom EUV mirror parameters

Similar to above but with user-defined delta and beta values for the complex refractive index:

n = (1 - delta) + i*beta

### LangasiteOnSilicon.fsx

Dispersive active crystal (Langasite) on Silicon substrate

Langasite (La3Ga5SiO14) is an optically active piezoelectric crystal used in sensors. This example: - Uses full dispersive optical properties from literature - Shows thin film vs. thick substrate comparison - Demonstrates wavelength-dependent behavior (250-600nm)

### ActiveCrystal.fsx

Magneto-optically active planar crystal plate

Demonstrates optical activity (polarization rotation) in crystals with non-zero rho tensor: - Uniaxial crystal (n11=2.315, n33=2.226) - Variable gyration coefficient g12 - Plots s- and p-polarized transmission vs. g12

### ActiveCrystalComparison.fsx

Comparison of different active crystal types

Compares three crystal symmetry classes: - Planar crystal: rho has only g12 component - Type 3,4,6 crystal: rho has g11 and g33 components - Type 32,42,62 crystal: rho has g11, g12, and g33 components

Shows transmitted azimuth and ellipticity as functions of incidence angle.

### TotalRefl_Glass.fsx

Total internal reflection at glass/vacuum interface

Light traveling from glass (n=1.50) to vacuum demonstrates: - Critical angle calculation - Brewster angle identification - s- and p-polarized reflectance behavior

### Wedge_Glass.fsx

Glass wedge prism

Analyzes a glass prism with 40-degree wedge angle: - Reflectance and transmittance vs. wedge angle (0-85 degrees) - Handles multiple internal reflections

### Wedge_BiaxialCrystal.fsx

Biaxial crystal wedge

Similar to glass wedge but with anisotropic biaxial crystal (nx=1.5, ny=1.65, nz=1.75): - Demonstrates birefringence effects in wedge geometry - 45-degree polarized input to excite both eigenmodes

### Wedge_ActiveCrystal.fsx

Active planar crystal wedge

Combines wedge geometry with optical activity: - 23-degree wedge of planar active crystal - Variable gyration coefficient analysis - Useful for polarimetric device design

### Wedge_ActiveCrystalComparison.fsx

Comparison of active crystal wedges

Compares wedge behavior for three crystal symmetry classes: - Transmitted intensity and polarization state - Azimuth and ellipticity of transmitted light - 23-degree wedge geometry

## API Reference

### Key Types

| Type | Description |
| --- | --- |
| WaveLength | Wavelength with units (nm, um, mm) |
| Thickness | Layer thickness with units |
| IncidenceAngle | Angle of incidence |
| Polarization | Linear polarization angle |
| Ellipticity | Elliptical polarization parameter |
| OpticalProperties | Material properties (eps, mu, rho) |
| Film | Thin film layer |
| OpticalSystem | Complete optical system |
| IncidentLightInfo | Light source specification |

### Key Functions

// Create optical system info
let info = { incidentLightInfo = light; opticalSystem = system.dispersive }

// Plot results
plot info [ R; T ] incidenceAngleRange
plot info [ Rs; Rp; Ts; Tp ] wavelengthRange
plotComparison [ system1; system2 ] [ R; T ] variable
plot3D info [ R ] xRange yRange

## Dependencies

- MathNet.Numerics: Linear algebra and numerical methods (customized version included)

- Plotly.NET: Interactive charting

- FSharp.Collections.ParallelSeq: Parallel processing

- xUnit: Testing framework

- FsCheck: Property-based testing

- FluentAssertions: Test assertions

## License

This project is licensed under the GNU General Public License v3.0. See the LICENSE file for details.

Individual components may have different licenses: - MatrixExp.fs: LGPL (ported from C code) - Core library packages: MIT license

## Author

Konstantin Konstantinov Softellect Systems, Inc.

## Repository

https://github.com/kkkmail/Berreman

## References

The Berreman matrix method is described in: - D. W. Berreman, “Optics in Stratified and Anisotropic Media: 4x4-Matrix Formulation,” J. Opt. Soc. Am. 62, 502-510 (1972)

## Version

Current version: 9.0.100.5 (targeting .NET 9.0)